### Electrical and Electronic System Design

The tile uses an Axial Flux Permanent Magnet (AFPM) generator instead of a radial flux design, as AFPM provides higher power density suitable for floor tiles. A planetary gear mechanism converts low-speed footstep motion into high-speed rotation up to 3300 RPM. According to Faraday’s Law, the induced voltage is proportional to angular velocity:

E_{max}=N*B*A*omega

The 8-pole, 9-coil three-phase stator generates about 120 V RMS and up to 50 W transient power. Since the output is high-frequency AC (220 Hz), a 3-phase bridge rectifier and synchronous buck converter are used to produce stable 12–24 V DC for charging batteries or supercapacitors.

The generated three-phase AC output is first converted into DC using a full-bridge rectifier and smoothing capacitor network. The rectified high-voltage DC is then regulated by an LTC3703-5 synchronous buck converter, which efficiently steps the fluctuating input down to a stable 12 V DC output suitable for energy storage and practical low-voltage applications.